| I. Name of the project | Oyu Tolgoi (Turquoise Hill) |  |
| --- | --- | --- |
| II. Sector | Mining |  |
| III. Project life, location | According to the Investment Agreement 30+20+20 years, Umnugobi province, Khanbogd soum |  |
| IV. Total investment | US $5.1 billion 2002-2009 – US$1 billion 2009-2013 – US$4 billion |  |
| V. Payback period | 5-8 years |  |
| VI. Project rationale | Government’s Action Plan Investment agreement |  |
| VII. Project goal, demand | Development of Oyu Tolgoi Gold and Copper Deposit |  |
| VIII. Project’s socio-economic benefit | - Development of South Gobi region - Benefit for foreign investors |  |
| IX. Required infrastructure | - Power plant - Diesel generator - Water treatment - Housing - Service and maintenance center - Office building - Checking point - Airstrip - Other facilities |  |
| X. Exploration history | As a result of extensive drilling and exploration work since 2000, high grade copper and gold resources were discovered in southern deposits (open-pit) and Hugo Dummett deposit. The Oyu Tolgoi deposits contain a total ore reserve of about 2.7 billion tones including 1028 tones of gold and more than 25.4 million tones of copper in resources in the Measured-and-Indicated, plus Inferred, categories. In 2005, an Integrated Development Plan (feasibility study) was presented. A new version of Integrated Development Plan will be presented in 2009-2010. |  |
| XI. Production capacity, technology | Concentrator with a single SAG circuit has the capacity of an initial throughput rate of 35 million tons (100,000 tons-per-day (tpd)).Assuming that the expansion is undertaken as scheduled, the project has the potential to achieve a mill throughput of 56 million tons per year (150,000 tpd). Initial, large open-pit mine will be developed on the near-surface Southern Oyu deposits; and underground block-cave mining operation at the Hugo Dummett North and South deposits is planned. The independent study indicates that the Oyu Tolgoi Mine will be capable of average annual production in excess of one billion pounds of copper and 330,000 ounces of gold for at least 35 years. Peak annual production in excess of 1.6 billion pounds of copper and 900,000 ounces of gold is projected to be reached six years after initial production begins (Year 6). |  |
| XII. Environmental impact | The environmental baseline study, Environmental Impact Assessment (EIA) and Environmental Protection Plan are in full compliance with accepted international standards for the mining industry, all environmental requirements of Mongolian laws and established standards, and standards established by The World Bank for international mineral development projects. IMMI is committed to achieving the international environmental management standards for the Oyu Tolgoi mine development. A Preliminary Assessment Report completed in early 2004 was prepared in conformance with ISO 14001 requirements. |  |
| XIII. Project personal | Name | Batkhuu G. |
|  | Job title | Director, Department of Mining and Heavy Industry |
|  | Organization name | Ministry of Mineral Recourses and Energy |
|  | Phone | 976-51-264059 |
|  | E-mail | batkhuu@mmre.energy.mn |
|  | Fax | 976-11-329222 |
|  | Name | Tsogtbaatar Ch. |
|  | Job title | Director, Mining and Heavy Industry Policy Department |
|  | Organization name | Ministry of Mineral Recourses and Energy |
|  | Phone | 976-262120, 976- 99117671 |
|  | E-mail | tsogtbaatar@mmre.energy.mn |
|  | Fax | 976-11-329222 |